BEFORE STARTING

Avionics Off

Safety Belts ON

Check Controls

TRIM - Takeoff

STARTING

Fuel ON

Mixture RICH

Prime (if necessary)

Carb Heat OFF

Master Switch ON

Throttle Cracked 1/4"

Call  'CLEAR'

Brakes / Yoke Back

Engage Starter

WARM UP

800-1200 rpm max 2-4 min.

Check Oil Pressure in Green

Mixture - Lean as needed

Transponder STANDBY

Avionics ON

Set Altimiter

Lights AS NEEDED

BEFORE TAKE-OFF

Check Controls

Run -up at 1,800 RPM

Check Mags  ( < 175 RPM )

Check Carb Heat

Oil Pressure

Oil Temp

Check Ammeter

Retard Throttle

Check Mag Grounding

TRIM - Takeoff

Mixture RICH

Fuel ON

Transponder ALT

LEVEL OFF

Set DG / AH

LANDING APPROACH

Mixture RICH

Carb Heat ON if needed

AFTER LANDING

Carb Heat OFF

SHUT DOWN

Transponder OFF

Avionics OFF

Lights OFF

Mixture IDLE CUT-OFF

Mags OFF

Master OFF

Fuel OFF

Secure Controls