1945 Taylorcraft BC12-D

N39240

Preflight:

- General Aircraft Condition

- Remove Gust-Locks

- Secure Tail w/ Locking Tow Rope

- Check Fuselage Fabric Condition

- Lt. Horizontal Stabilizer Security

- Lt. Tail Brace Wires and Clevises

- Lt. Elevator Hinges and Pins

- Elevator Trim Tab

- Vertical Stabilizer Security

- Rudder Hinges and Pins

- Tail Wheel Springs, Tire, Fasteners

- Check Condition of Wing Fabric

- Rt. Elevator Hinges and Pins

- Rt. Tail Brace Wires and Clevises

- Rt. Horizontal Stabilizer Security

- Rt. Wing Root Fairing

- Rt. Aileron Hinges, Pins, Control Horn

- Rt. Wing Tip Flex Spar Check

- Rt. Wing Leading Edge

- Rt. Fuel Cap and Vent

Start-Up:

- Set Brakes

- Brief Passenger Ignition/Throttle

- Ensure Tail is Tied / Chock Wheels

- Ensure Aircraft Will Not Move

- Ignition Off / Fuel On

- Pull Prop Thru 4 Blades

- Prime as Needed

- Throttle Cracked 1/8”

- Fuel Off

- Ignition On

- Rapidly Pull Prop Thru

- Walk Around Wing Tip to Cabin

  - Remain Clear of Prop Arc

- RPM 1000 or Less

- Oil Pressure 30 p.s.i. Normal

- After Engine Stabilizes, Untie Tail

- Fuel On and Locked in Position

- Secure Doors/Windows

- Secure Passenger Safety Belts

Before Flight:

- Set Brakes

- Check Flight Controls Free and Correct

- Check Tail for Water

- Set Altimeter

- Turn Coordinator Ball Free

- Pitch Trim Set

- Check Fuel Quantity

- Rt. Wing Strut Fittings

- Rt. Wing Jury Strut

- Rt. Wing Fuel Sump (if Fueled)

- Rt. Gear Attachments, Tire, Fasteners

- Cowling and Undercarriage Condition

- Check Oil and Ignition Wires

- Check Carb. and Cowling Ventilation

- Check Prop/Spinner Condition

- Check Fuel Cap Tight, Wire Free

- Sump Fuel, Check Ignition Wires

- Lt. Gear Attachments, Tire, Fasteners

- Lt. Wing Jury Strut

- Lt. Wing Strut Fittings

- Lt. Wing Fuel Sump (if Fueled)

- Lt. Fuel Cap and Vent

- Lt. Wing Leading Edge

- Pitot Tube Clear

- Lt. Wing Tip Flex Spar Check

- Lt. Aileron Hinges, Pins, Control Horn

- Lt. Wing Root Fairing

- Check Control Cables Rear Cockpit

- Check Magnetos L & R

  - 100 rpm Max Drop Each

- Check Carburetor Heat

  - 100 RPM Max Drop Each

- Primer Locked

- Fuel Valve Locked

- Release Brakes

Flight Performance:

Vs			37 mph

Vx			55 mph

Vy			65 mph

Best Glide		60 mph

Minimum Approach	50 mph

Vr			50 mph

Vne			140 mph

Specifications:

Empty Weight		800 lbs

Gross Weight		1200 lbs

Max Fuel 		24 gals

Normal Fuel 		12 gals

Oil Capacity		4 qts