I’M SAFE											 AWOS

Illness?					Ames			515-233-2611		Ankeny			515-965-5764

Medication?			Boone			515-432-9042		Chariton		641-774-5645

Stress?					DSM AWOS	515-287-1012		DSM ATIS		515-974-8046

Alcohol?				Grinnell		641-236-9720		Knoxville		641-828-8406

Fatigue?				Marshalltown	641-752-2339		Newton			641-791-3302

Eaten?					Pella			641-628-3459		Perry			515-465-2269

WEATHER BRIEFING SEQUENCE

1-800-WX -BRIEF (992-7433)								CST	CDT	UTC

1 = briefer / 2 = TIBS / 3= Fast File / 42+2 = Iowa

24:00	01:00	06:00

- Type of briefing: standard/abbreviated/outlook			01:00	02:00	07:00

- VFR or IFR												02:00	03:00	08:00

- Aircraft type and identification							03:00	04:00	09:00

- Departure airport										04:00	05:00	10:00

- Proposed route											05:00	06:00	11:00

- Destination airport										06:00	07:00	12:00

- Flight altitude											07:00	08:00	13:00

- Estimated time of departure								08:00	09:00	14:00

- Estimated time enroute									09:00	10:00	15:00

- Other NOTAMs? Any TFRs?							10:00	11:00	16:00

11:00	12:00	17:00

FLIGHT PLAN SEQUENCE								12:00	13:00	18:00

1-800-WX -BRIEF (992-7433)								13:00	14:00	19:00

14:00	15:00	20:00

- Type of plan (VFR/IFR)									15:00	16:00	21:00

- Aircraft identification									16:00	17:00	22:00

- Aircraft type / special equipment (TAYB/X)				17:00	18:00	23:00

- True airspeed (knots)									18:00	19:00	24:00

- Departure point											19:00	20:00	01:00

- Departure time (Zulu)									20:00	21:00	02:00

- Cruising altitude										21:00	22:00	03:00

- Route of flight											22:00	23:00	04:00

- Destination (airport & city)								23:00	24:00	05:00

- Estimated time enroute: hours & minutes				24:00	01:00	06:00

- Remarks

- Fuel on board: hours & minutes

- Alternate airport

- Pilot's name, address, phone number and home base

- Number of people aboard

- Color of aircraft (W/B)

- Destination contact and phone number (optional)

Des Moines Tower		515-287-4620

Des Moines TRACON	515-287-1687

Exec 1 Aviation			515-965-1020

IFR/SVFR Clearances	888-766-8267

Iowa FSDO				515-289-3840

TIBS					877-484-2799 (Iowa 42+2)

TAYLORCRAFT BC12-D SPECIFICATIONS

Cruise					  80 mph

VNO					105

VNE						140

VS						  40

VX	best angle of climb	  55

VY	best rate of climb	  65

Best glide				  60

Approach				  55

Fuel capacity main		  12 gal

wing		    6 gal

Fuel consumption		    4.25 gph

Oil capacity				   4 qts

Tire pressure			  20 psi

Engine

Continental A-65-8

horsepower			65 hp @ 2300 rpm

max engine speed	2300 rpm

cruise				2150 rpm

Length					21' 9"

Height					  6' 6"

Wingspan				36'

Wing chord				  5' 3"

NC96130 WEIGHT & BALANCE

Max weight				1200 lbs.

Empty weight			  793

Max useful load			  407

Max baggage			    50

C.G. limits (datum is leading edge of wing)

forward				+14.2 in.

aft					+20.0 in.

weight     x     arm     =     moment

EW (includes 4 qts. oil)	793 lbs.		+14.53 in.	11,525 in.-lbs.	(oil 1.875 lbs./qt.)

Nose fuel							–  9 in.						(avgas 6 lbs./gal.)

Wing fuel							+ 24 in.

Seats								+ 23 in.

Baggage compartment				+ 40 in.

Total moments (in.-lbs.) ÷ total weights (lbs.) = center of gravity (in.)

tail rope  3.5 lbs.

tools & manuals bag  7 lbs.

pilot bag, radio, headset & intercom  8 lbs.

tiedowns kit  10 lbs.

TAYLORCRAFT BC12-D CHECKLISTS

PREFLIGHT INSPECTION

CABIN

- Pilot papers … CHECK pilot and medical certificates, logbook in plane

- Aircraft papers ... airworthiness, registration, operation limitations, weight & balance

- Tie-down kit, oil, cleaning supplies ... CHECK in baggage compartment if needed

- Maintenance, inspections ... CHECK when oil change, lube, ELT battery, annual due

- Fire extinguisher ... CHECK secure

- ELT … CHECK secure, armed

- Rudder pedals, brake pedals, control cables, pulleys ... CHECK

- Magneto switch ... OFF

- Parking brake ... OFF

- Carburetor heat ... OFF

- Fuel valve ... ON

- Throttle ... CLOSED

- Primer ... IN

- Wing tank fuel transfer valve ... OFF

- Elevator trim ... SET NEUTRAL 2½ crank turns from either stop

- Trim tab ... LOOK to verify neutral

LEFT SIDE OF FUSELAGE

- Windows ... CHECK

- Door ... CHECK hinges

- Wing root fairing … CHECK

- Fuselage ... CHECK top, side, bottom

EMPENNAGE

- Tie-down rope, anchor ... UNTIE and stow

- Left stabilizer ... CHECK nuts

- Brace wires ... CHECK bolts, tight, low bass tone

- Left elevator ... CHECK hinges, pins, movement

- Trim tab ... CHECK actuating rod, nut, pins

- Elevator bellcrank inspection hole covers … CHECK secure

- Tailwheel spring … CHECK bolts, pins

- Tailwheel ... CHECK tread, springs, chains, links, arms, bolts, pins

- Rudder ... CHECK cables, nuts, pins, safety wire, stops, hinges, pins, movement, trim tab

- Right elevator ... CHECK bellcrank nut, pin hinges, pins, movement

- Brace wires ... CHECK bolts, tight, low bass tone

- Right stabilizer ... CHECK nuts

RIGHT SIDE OF FUSELAGE

- Fuselage ... CHECK top, side, bottom

- Wing root fairing … CHECK

- Windows ... CHECK

- Door ... CHECK hinges

TAYLORCRAFT BC12-D CHECKLISTS

PREFLIGHT INSPECTION

RIGHT WING

- Fuel tank sump quick-drain valve ... DRAIN fuel to check for water, sediment,													and proper fuel grade (100 LL blue);															CHECK no leakage from valve

- Fabric (top and bottom) ... CHECK for wrinkles, tears, frost

- Inspection hole covers ... CHECK secure

- Struts ... CHECK straight, no bends, bolts, pins

- Aileron ... CHECK hinges, pins, bolts, bellcrank, other aileron moves opposite direction

- Aileron bellcrank inspection panel ... CHECK secure

- Wingtip ... CHECK

- Leading edge ... CHECK

- Tie-down rope, anchor ... UNTIE and stow

- Wing root fairing ... CHECK

- Fuel quantity ... CHECK with dipstick

- Fuel tank cap ... SECURE

- Fuel tank cap vent tube ... CHECK unobstructed, facing forward

RIGHT MAIN LANDING GEAR

- Struts ... CHECK bungee cover secure, no bends, bolts, pins

- Brake... CHECK spring, hinge pins, adjustment nut clips

- Tire ... CHECK inflation, tread

- Hubcap ... CHECK bolts

NOSE

- Windshield ... WIPE clean, CHECK for cracks

- Fuel quantity … CHECK with dipstick

- Fuel tank cap ... SECURE

- Fuel gauge wire ... CHECK movement

- Right cowling … OPEN

- Magnetos ... CHECK ignition wires secure

- Engine oil dipstick/filler cap ... CHECK (4 quarts)

- Engine oil dipstick/filler cap ... SECURE

- Right cowling … LATCH, REPLACE cotter pins

- Crankshaft seal … CHECK no oil leaking

- Propeller ... CHECK for nicks or damage

- Spinner ... CHECK secure

- Engine cooling air inlets ... CHECK clear of obstructions

- Carburetor air scoop ... CHECK clear of obstructions

- Left cowling … OPEN

- Gascolator quick-drain valve ... DRAIN fuel to check for water, sediment, 													and proper fuel grade (100 LL blue); 														CHECK no leakage from valve

- Left cowling … LATCH, REPLACE cotter pins

TAYLORCRAFT BC12-D CHECKLISTS

PREFLIGHT INSPECTION

LEFT MAIN LANDING GEAR

- Struts ... CHECK bungee cover secure, no bends, bolts, pins

- Brake... CHECK spring, hinge pins, adjustment nut clips

- Tire ... CHECK inflation, tread

- Hubcap ... CHECK bolts

LEFT WING

- Wing root fairing ... CHECK

- Pitot tube ... CHECK cover pivots, CHECK clear of obstructions

- Tie-down rope, anchor … UNTIE and stow

- Leading edge ... CHECK

- Wingtip … CHECK

- Aileron ... CHECK hinges, pins, bolts, bellcrank, other aileron moves opposite direction

- Aileron bellcrank inspection panel ... CHECK secure

- Struts ... CHECK straight, no bends, bolts, pins

- Inspection hole covers ... CHECK secure

- Fabric (top and bottom) ... CHECK for wrinkles, tears, frost

BEFORE STARTING ENGINE

- Preflight inspection … COMPLETE

- Passenger briefing … COMPLETE

- Radio, intercom, PTT switch, headset … CONNECTED

- Charts, kneeboard, pen, sunglasses … READY

- Fuel sumps … SAMPLED

- Oil dipstick … CHECKED

TAYLORCRAFT BC12-D CHECKLISTS

ENGINE START

STARTING ENGINE ALONE

- Mental caution … propeller can kill you or someone else

- Run-away plane path … CHECK blocked with something to stop run-away plane

- Tail tie-down rope ... TIED, no slack

- Chocks ... SET, lines tied inside cabin

- Parking brake … SET

- Magneto switch … OFF

- Carburetor heat … OFF

- Fuel valve … ON

- Throttle … CLOSED, TIGHTENED

- Control wheel ... TIE BACK with seat belt

- Hat, sunglasses … REMOVE and STOW

- “BRAKES, SWITCH OFF, THROTTLE CLOSED!”

- Engine … LIMBER and PRIME:

cold engine, cold day: pull through 10 blades, 2 shots prime, 10 blades, 1 shot prime

warm engine, cold day: 5 blades, 2 shots prime, 5 blades

cold engine, warm day: 5 blades, 1 shot prime, 5 blades

warm engine, warm day: no prime, 5 blades

- Fuel valve … OFF

- “BRAKES, CONTACT, THROTTLE CRACKED!”

- Footing … SECURE

- Propeller … PULL THROUGH

- Throttle ... SET for 700 rpm

- Oil pressure ... CHECK 10 – 35 psi within 30 seconds

- Fuel valve … ON

- Control wheel ... RELEASE seat belt

- Chocks ... PULL and STOW

- Tail tie-down rope ... RELEASE and STOW

STARTING ENGINE WITH HELP

- Tie-down ropes ... REMOVED and STOWED

- Brakes … ON

- Magneto switch … OFF

- Carburetor heat … OFF

- Fuel valve … ON

- Throttle … CLOSED, TIGHTENED

- “BRAKES, SWITCH OFF, THROTTLE CLOSED!”

- Engine … LIMBER and PRIME:

cold engine, cold day: pull through 10 blades, 2 shots prime, 10 blades, 1 shot prime

warm engine, cold day: 5 blades, 2 shots prime, 5 blades

cold engine, warm day: 5 blades, 1 shot prime, 5 blades

warm engine, warm day: no prime, 5 blades

- Fuel valve … OFF

- Control wheel … FULL BACK

- “BRAKES, CONTACT, THROTTLE CRACKED!”

- Propeller … PULL THROUGH

- Throttle ... SET for 700 rpm

- Oil pressure ... CHECK 10 – 35 psi within 30 seconds

- Fuel valve … ON

TAYLORCRAFT BC12-D CHECKLISTS

TAXI, RUN-UP

BEFORE TAXIING

- Engine start time … NOTE

- Radio ... AWOS/ATIS

- Wing tank fuel transfer valve ... OFF

- Elevator trim crank ... NEUTRAL

- Compass ... CHECK fluid full, no leaks

- Airspeed indicator ... CHECK on 0 mph

- Tachometer ... CHECK

- Altimeter ... SET station pressure or field elevation, CHECK for error

- Oil temperature gauge ... CHECK within 90 – 212 degrees

- Oil pressure gauge ... CHECK within 30 – 40 psi

- Magneto switch ... CHECK on BOTH

- Carburetor heat ... OFF unless needed

- Fuel valve ... ON

- Primer ... IN

- Cabin heat ... SET as needed

- Radio ... Ground control or self-announce

TAXI

- Brakes ... RELEASE and CHECK after rolling

- Controls ... POSITION for wind (climb into, dive away)

- Compass ... CHECK swings freely

RUN-UP

- Brakes ... SET

- Seat belts ... CHECK secure

- Doors ... CLOSED and latched

- Controls ... FREE and correct

- Control wheel ... FULL BACK

- Throttle ... 1500 rpm

- Oil temperature gauge ... CHECK within 90 – 212 degrees

- Oil pressure gauge ... CHECK within 30 – 40 psi

- Magnetos ... CHECK left/both/right/both (max drop 75 rpm)

- Carburetor heat ... ON, CHECK for 75 – 200 rpm drop

- Throttle ... CHECK low idle with carb heat

- Carburetor heat ... OFF

- Throttle ... 800 rpm

- Windsock ... CHECK wind direction and speed

- Self-brief ... abort and engine failure procedures

- Other aircraft ... LOOK both directions, LOOK AGAIN

- Radio ... tower or self-announce

- Takeoff time ... NOTE

- Brakes ... RELEASE

TAYLORCRAFT BC12-D CHECKLISTS

NORMAL PROCEDURES

CROSSWIND TAKEOFF

- Wings ... AILERON INTO WIND

- Throttle ... FULL OPEN

- Oil pressure gauge … CHECK within 30 – 40 psi

- Airspeed indicator … CHECK active

- Rotation ... Keep airplane firmly on runway until 60 mph, then rotate

- Climb ... After lift-off establish wind correction angle

SOFT FIELD TAKEOFF

- Throttle ... FULL OPEN

- Oil pressure gauge … CHECK within 30 – 40 psi

- Airspeed indicator … CHECK active

- Elevator ... Lift tail as soon as possible

- Lift-off ... In ground effect

- Climb ... Remain in ground effect until 65 mph, then climb

SHORT FIELD TAKEOFF

- Carburetor heat ... OFF

- Runway ... Start at very beginning

- Brakes ... APPLY

- Throttle ... FULL OPEN

- Brakes ... RELEASE

- Oil pressure gauge … CHECK within 30 – 40 psi

- Airspeed indicator … CHECK active

- Elevator ... SLIGHTLY TAIL LOW

- Rotation ... LIFT NOSE at 60 mph

- Climb ... 55 mph until obstacles cleared then pitch for 65 mph

TAYLORCRAFT BC12-D CHECKLISTS

NORMAL PROCEDURES

LANDING

- Carburetor heat … ON

- Wing tank fuel transfer valve … OFF

- Seat belts, shoulder harnesses … SECURE

- Throttle … REDUCE abeam landing point

- Elevator trim … ADJUST nose-up as needed

- Airspeed … 55 mph

CLEAR OF RUNWAY

- Carburetor heat … OFF

- Elevator trim … RE-SET to neutral

ENGINE SHUTDOWN

- Brakes … SET

- Throttle … IDLE

- Magneto P-lead grounding … CHECK mag switch both-off-both

- Magneto switch … OFF

- Throttle … OPEN, then CLOSE

- Radio, intercom … OFF

- Time … NOTE

MOORING

- Control wheel … TIE BACK with seatbelt

- Chocks … SECURE

- Tie-down ropes … SECURE

TAYLORCRAFT BC12-D CHECKLISTS

EMERGENCY PROCEDURES

SEVERE TURBULENCE

- Airspeed ... REDUCE TO 95 MPH

ENGINE EMERGENCY SHUT DOWN

- Throttle ... IDLE

- Magneto switch ... OFF

- Fuel valve ... OFF

ENGINE FAILURE DURING TAKEOFF ROLL

- Throttle ... IDLE

- Brakes ... APPLY

- Magneto switch ... OFF

- Fuel valve ... OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed ... BEST GLIDE 60 MPH

- Landing ... STRAIGHT AHEAD or slight direction change, only enough to clear obstacles

PARTIAL POWER FAILURE DURING FLIGHT

- Carburetor heat ... ON

- Airspeed ... AS REQUIRED TO HOLD ALTITUDE

- Fuel valve ... CHECK ON

- Magneto switch ... CHECK ON BOTH

- Primer … CHECK IN

- Land ... AT NEAREST AIRPORT

- NOTE:	If power available is insufficient to maintain level flight, select a field as a possible 	forced landing site and proceed as follows:

- Airspeed ... BEST GLIDE 60 MPH

- Throttle ... ALL AVAILABLE POWER until a safe landing is assured

TAYLORCRAFT BC12-D CHECKLISTS

EMERGENCY PROCEDURES

EMERGENCY LANDING WITHOUT ENGINE POWER

- Seatbelts ... SECURE

- Airspeed ... BEST GLIDE 60 MPH

- Select field ... CLEAR, FIRM, and INTO THE WIND

- Radio ... TRANSMIT MAYDAY on 121.5 Mhz, giving location and intentions

- Magneto switch ... OFF

- Fuel valve ... OFF

- Doors ... UNLATCHED PRIOR TO TOUCHDOWN

FIRE DURING ENGINE START ON GROUND

- If engine starts:

- Throttle ... 1500 RPM for a few minutes

- Engine ... SHUT DOWN and inspect for damage

- If engine fails to start:

- Throttle ... FULL OPEN

- Magneto switch … OFF

- Fuel valve ... OFF

- Fire extinguisher ... ACTIVATE

ENGINE FIRE IN FLIGHT

- Cabin heat ... OFF

- Windows ... OPEN

- Airspeed ... 90 mph (If fire is not extinguished, increase glide speed to find an airspeed

- which will provide an incombustible mixture)

- NOTE: If fire continues, a slip may help keep fire from the cabin area

- Forced landing ... EXECUTE

LOW OIL PRESSURE OR HIGH OIL TEMPERATURE

1.	Throttle ... 2150 rpm

2.	Airspeed ... CRUISE

3.	Carburetor heat ... OFF

If condition continues:

4.	Land ... as soon as practical