Appendix D to Part 43—Scope and Detail of Items (as Applicable to the Particular Aircraft)

To Be Included in Annual and 100-Hour Inspections

(a) Each person performing an annual or 100-hour

inspection shall, before that inspection, remove

or open all necessary inspection plates, access

doors, fairing, and cowling. He shall thoroughly

clean the aircraft and aircraft engine.

(b) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) the

following components of the fuselage and hull

group:

(1) Fabric and skin—for deterioration,

distortion, other evidence of failure, and

defective or insecure attachment of fittings.

(2) Systems and components—for improper

installation, apparent defects, and

unsatisfactory operation.

(3) Envelope, gas bags, ballast tanks, and

related parts—for poor condition.

(c) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) the

following components of the cabin and cockpit

group:

(1) Generally—for uncleanliness and loose

equipment that might foul the controls.

(2) Seats and safety belts—for poor condition

and apparent defects.

(3) Windows and windshields—for

deterioration and breakage.

(4) Instruments—for poor condition,

mounting, marking, and (where practicable)

improper operation.

(5) Flight and engine controls—for improper

installation and improper operation.

(6) Batteries—for improper installation and

improper charge.

(7) All systems—for improper installation,

poor general condition, apparent and

obvious defects, and insecurity of

attachment.

(d) Each person performing an annual or 100-hour

inspection shall inspect (where applicable)

components of the engine and nacelle group as

follows:

(1) Engine section—for visual evidence of

excessive oil, fuel, or hydraulic leaks, and

sources of such leaks.

(2) Studs and nuts—for improper torquing

and obvious defects.

(3) Internal engine—for cylinder compression

and for metal particles or foreign matter on

screens and sump drain plugs. If there is

weak cylinder compression, for improper

internal condition and improper internal

tolerances.

4) Engine mount—for cracks, looseness

of mounting, and looseness of engine to

mount.

5) Flexible vibration dampeners—for poor

condition and deterioration.

6) Engine controls—for defects, improper

travel, and improper safetying.

7) Lines, hoses, and clamps—for leaks,

improper condition, and looseness.

8) Exhaust stacks—for cracks, defects, and

improper attachment.

9) Accessories—for apparent defects in

security of mounting.

10) All systems—for improper installation,

poor general condition, defects, and

insecure attachment.

11) Cowling—for cracks and defects.

Each person performing an annual or 100-hour

inspection shall inspect (where applicable)

the following components of the landing gear

group:

1) All units—for poor con

dition andinsecurity of attachment.

2) Shock absorbing devices—for improper

oleo fluid level.

3) Linkages, trusses, and members—for

undue or excessive wear fatigue, and

distortion.

4) Retracting and locking mechanism—for

improper operation.

5) Hydraulic lines—for leakage.

6) Electrical system—for chafing and

improper operation of switches.

7) Wheels—for cracks, defects, and

condition of bearings.

8) Tires—for wear and cuts.

9) Brakes—for improper adjustment.

10) Floats and skis—for insecure attachment

and obvious or apparent defects.

(f) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) all

components of the wing and center section

assembly for poor general condition, fabric or

skin deterioration, distortion, evidence of failure,

and insecurity of attachment.

(g) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) all

components and systems that make up the complete

empennage assembly for poor general

condition, fabric or skin deterioration, distortion,

evidence of failure, insecure attachment, improper

component installation, and improper component

operation.

(h) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) the

following components of the propeller group:

(1) Propeller assembly—for cracks, nicks,

binds, and oil leakage.

(2) Bolts—for improper torquing and lack of

safetying.

(3) Anti-icing devices—for improper

operations and obvious defects.

(4) Control mechanisms—for improper

operation, insecure mounting, and restricted

travel.

(i) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) the

following components of the radio group:

(1) Radio and electronic equipment—for

improper installation and insecure

mounting.

(2) Wiring and conduits—for improper

routing, insecure mounting, and obvious

defects.

(3) Bonding and shielding—for improper

installation and poor condition.

(4) Antenna including trailing antenna—for

poor condition, insecure mounting, and

improper operation.

(j) Each person performing an annual or 100-hour

inspection shall inspect (where applicable) each

installed miscellaneous item that is not otherwise

covered by this listing for improper installation

and improper operation.