| MAJOR REPAIR AND ALTERATION (Airframe, Powerplant, Propeller, or Appliance) U.S Department of Transportation Federal Aviation Administration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Form Approved OMB No. 2120-0020 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # For FAA Use Only |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Office Identification |  |  |
| INSTRUCTIONS: Print or type all entries. See FAR 43.9, FAR 43 Appendix B, and AC 43.9-1 (or subsequent revision thereof) for instructions and disposition of this form. This report is required by law (49 U.S.C. 1421). Failure to report can result in a civil penalty not to exceed $1,000 for each such violation (Section 901 Federal Aviation Act 1958) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Aircraft |  |  |  | Make Taylorcraft |  |  |  |  |  |  |  | Model BC12-D |  |  |  |  |  |  |
|  |  |  |  | Serial No. 7040 |  |  |  |  |  |  |  | Nationality and Registration Mark N43381 |  |  |  |  |  |  |
| 2. Owner |  |  |  | Name (As shown on registration certificate) Richard Pearson |  |  |  |  |  |  |  | Address (As shown on registration certificate) PO Box 55483 Portland, OR 97238 |  |  |  |  |  |  |
| 3. For FAA Use Only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Unit Identification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5. Type |  |
| Unit |  |  | Make |  |  |  |  | Model |  |  |  |  | Serial No. |  |  |  | Repair | Alteration |
| AIRFRAME |  |  | ~~~~~~~~~~~~~~~~~~~~(As described in item 1 above)~~~~~~~~~~~~~~~~~~~~ |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |
| POWERPLANT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROPELLER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APPLIANCE |  |  | Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Manufacturer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. Conformity Statement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A. Agency’s Name and Address |  |  |  |  |  |  |  |  |  | B. Kind of Agency |  |  |  |  | C. Certificate No. |  |  |  |
| Darrell Walkemeyer 3 |  |  |  |  |  |  |  |  |  |  | U.S. Certificated Mechanic |  |  |  | A/P 1062920 |  |  |  |
| 12207 NE 273rd Street |  |  |  |  |  |  |  |  |  |  | Foreign Certificated Mechanic |  |  |  |  |  |  |  |
| Battle Ground, WA 98604 |  |  |  |  |  |  |  |  |  |  | Certificated Repair Station |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Manufacturer |  |  |  |  |  |  |  |
| D. I certify that the repair and/or alteration made to the unit(s) identified in item 4 above and described on the reverse or attachments hereto have been made in accordance with the requirements of Part 43 of the U.S. Federal Aviation Regulations and that the information furnished herein is true and correct to the best of my knowledge. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date |  |  |  |  |  |  |  |  |  |  | Signature of Authorized Individual |  |  |  |  |  |  |  |
| 7. Approval for Return to Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pursuant to the authority given persons specified below, the unit identified in item 4 was inspected in the manner prescribed by the Administrator of the Federal Aviation Administration and is APPROVED REJECTED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BY |  | FAA Flt Standards Inspector |  |  |  |  | Manufacturer |  | x |  | Inspection Authorization |  |  | Other (Specify) |  |  |  |  |
|  |  | FAA Designee |  |  |  |  | Repair Station |  |  |  | Person Approved by Transport Canada Airworthiness Group |  |  |  |  |  |  |  |
| Date of Approval or Rejection |  |  |  |  |  | Certificate or Designation No. A/P-IA 1062920 |  |  |  |  | Signature of Authorized Individual |  |  |  |  |  |  |  |

FAA Form 337    (12-88)

NOTICE Weight and balance or operating limitation changes shall be entered in the appropriate aircraft record. An alteration must be compatible with all previous alterations to assure continued conformity with the applicable airworthiness requirements.

8. Description of Work Accomplished (If more space is required, attach additional sheets. Identify with aircraft nationality and registration mark and date work completed.)

1946 Taylorcraft BC12D, Serial No. 7040, N43381 Aircraft modified to provide maintenance access to wing fuel tanks by removing fabric covering over tanks and installing sheet aluminum removable access covers, See attached Drawing A for general layout and overview. Fabric over fuel tanks cut according to Drawing B. Fabricate crossmember, as shown in Drawing C, using procedures in AC43.13 paragraph 4-55. Prime all surfaces with zinc chromate primer. Attach to ribs, as shown in Drawing D, using four NAS1329A06K75, 6-32 rivnuts, two at each end. Attach fabric to leading edge skin, inside of ribs, and fabricated crossmember, as shown in Drawing B. Install NAS1329A06K75, 6-32 rivnuts in leading edge skin and two inboard ribs as shown in Drawing D. Fabricate fuel tank access covers using .025" 2024T3 aluminum sheet as shown in Drawing E. Prime all surfaces with zinc chromate primer. Upper surface finished to match airplane. Edge of sheet and inside of filler neck hole are edged with rubber U channel. Attach fuel tank access covers to top of wing using AN526-632R6 stainless steel screws. Change in weight and balance is negligible. No additional maintenance is due beyond the requirements of 14 CFR, Part 91, Subpart E. Instructions for Continued Airworthiness #1 This form 337 will become part of the aircrafts maintenance records. These ICA's will be incorporated into the aircraft annual inspection #2 Ensure fabric glue used in modification is approved for covering process, currently Air-Tech Coatings #3 Replaced fabric over wing tanks with .025" 2024T3 aluminum sheet #4 No special control or operation procedures #5 N/A #6 Inspect prior to each flight, & at annual inspection, for security or interference. Ensure legibility of fuel placards. #7 N/A #8 N/A #9 N/A #10 N/A #11 Refinish with protective coating as required to prevent corrosion. #12 Fasteners are NAS526-632R6C screws and NAS1329A06K75 rivnuts. Fabric glue as req by covering process. #13 N/A #14 N/A #15 N/A #16 N/A #17 For changes to the ICA's contained on this 337 a letter will be submitted to the FSDO for acceptance. Once accepted a log book entry will be entered in the aircraft logbooks identifying the revision, location, and date. END

Additional Sheets Are Attached

U.S.GPO:1989-0-663-171

U.S.GPO:1989-0-663-171