TAYLORCRAFT BC12-D INSPECTION INSTRUCTIONS

N_______

DATE:  ____/_______

ENGINE OPERATION:

__ -Run engine to minimum 120 degrees oil temperature - check full throttle static RPM

(Consult specifications for propeller used).  2070 rpm

__ -Check magnetos - 75 RPM drop at 1800 RPM.
__ -Check carburetor heat - 100 RPM drop at full throttle.
__ -Check ignition switch for operation.
__ -Check idle RPM 550 - 600 RPM with carburetor heat off.
__ -Oil pressure 10 - 35 lbs.

ENGINE MOUNTS AND ATTACHMENTS:

__ -Check engine mounts for damage and cracks at gussets or in corners.

__ -Inspect protective finish on mount; sand and touch up bare areas.

__ -Inspect rubber shock mounts for rubber deterioration and tension.

__ -Engine mount bolts should be tightened to 60 to 80 inch lbs.
__ -Check mount bolts for safety.

COWLING AND BAFFLES:

__ -Clean and inspect engine cowling for dents and cracks at hinges and reinforcement.       __ -Check for tension adjustment on cowl doors at fasteners.
__ -Check baffles for cracks and baffle seals for damage.

MAGNETOS, WIRING AND SHIELDING (IF INSTALLED):

__ -Check magnetos for secure attachment and oil leaks.
__ -Check insulation for deterioration and chafing.
__ -Check plug wiring connections at magneto and spark plugs.

__ -Check for grommets at baffles and at firewall.

OIL DRAIN AND SAFETY PLUG:

__ -Drain oil and check for metal particles.  Reinstall drain plug and safety.
__ -Remove, clean and check oil screen for metal particles.  Reinstall and safety.

__ -Check flexible lines for deterioration.  Check fittings for leaks.

COMPRESSION TEST RESULTS:

(1) -      /80

(2)-      /80

(3)-      /80

(4)-      /80

                                                       PAGE 1

SPARK PLUG SERVICE:

__ -Remove plugs, abrasive blast, remove any lead deposits and test.
__ -Plugs with badly burned electrodes should be replaced.
__ -Reset gap to .016-.021, REM40E and M41E plugs.
__ -Reinstall using thread lubricant and new gaskets to prevent leakage and seizing.

Torque to 300 to 360 inch lbs.  (25-30 ft lbs.)

CARBURETOR AND HEATER:

__ -Check carburetor for mounting security.
__ -Inspect carburetor bowl for cracks, particularly at inlet.
__ -Drain carburetor float chamber and check inlet finger screen safety.
__ -Operate throttle in cockpit to be sure that throttle arm hits stops in open and closed

positions without binding or sticking.  Lubricate rod, connecting linkage and

carburetor butterfly shafts.

__ -Mixture control wired to full rich position or block-off plate installed.
__ -Inspect carburetor air box for security and cracks - heater valve for full travel.

__ -Check rubber intake hose connections for deterioration and clamp security.
__ -Check intake system for leaks and cracks.
__ -Inspect air filter housing and replace air filter.
 
FUEL LINES AND STRAINER:

__ -Check fuel lines for leaks and hose deterioration.

__ -Check hose supports for security and chafing. Drain and clean fuel strainer and

safety.
__ -Check for stains around fuel system indicating leaks. Check all connections for

tightness.

EXHAUST STACK:

__ -Check stack flanges for security, cracks and leaks.
__ -Remove all heater shrouds and inspect for corrosion, cracks and leaks that might

transfer exhaust gas to the cockpit, particularly through the cabin heater system.
__ -Check tailpipe and stacks for security at all clamps and joints. Check cabin heater

box and control valve for operation.
__ -Check cabin and carburetor heat flexible tubing for security and general condition.

ENGINE CONTROLS AND FIREWALL:

__ -Check firewall for open holes and gas leaks from engine compartment. (If open

holes, use zinc chromate putty or some other recommended commercial brand.)
__ -Check all controls for grommets and sealing putty.

PROPELLER:

__ -Remove spinner and check for cracks or dents in spinner and back plates.
__ -Check propeller for separated laminations, cracks, loose metal tipping and protective

finish.

__ -Blade track within 1/16".
__ -Wood propeller hub bolts are to torque from 175 to 225 inch lbs.  Results: _____
                                                   
                                                         PAGE 2

COCKPIT AND BAGGAGE AREA:

__ -Seats: Check general condition of canvas sling and seat cushions.
__ -Check condition of safety belts, if frayed, replace. Check baggage area canvas, if

deteriorated or ripped, replace.

__ -Test ELT and check battery date.  Battery replacement due date:  ______________

WINDSHIELD:

__ -Check weather-stripping for security in channels and for leaks.
__ -Check plastic windshield and side windows for mounting hardware, cracks, crazing,

distortion and discoloration.

ENGINE INSTRUMENTS:

__ -Check engine instruments for mounting security. Check connections and plugs.
__ -Check placards and limitation markings.

__ -Tachometer: Red line - 2300 RPM
__ -Oil pressure: Red line 10 PSI
__ -Oil temperature: Red line - 220 degrees F.  Green arc - 120 to 220 degrees F.

Yellow arc - 40 to 120 degrees F.

FLIGHT INSTRUMENTS:

__ -Check flight instruments for mounting security. Check connections and plugs.
__ -Check placards and limitation markings.
__ -Air speed: Red line - 140 MPH Landplane 

DOOR LATCH AND HINGES:

__ -Check door hinge and rivets for looseness.  Lubricate hinges.
__ -Check door latch plunger for complete extension.  Lubricate.

__ -Check door for proper fit or damage resulting in air leaks.

ENGINE CONTROLS

__ -Check carburetor heat for panel placard and smoothness of operation. 
__ -Check throttle for smooth operation and operation of friction lock. 
__ -Check primer for operation and leaks behind the panel.
__ -Check cabin heat for panel placard and full travel of heater butterfly valve. 
__ -Check ignition switch for panel and terminal security.
__ -Check for placard - Off, left, right and both.

RUDDER PEDALS AND LINKAGE:

__ -Check rudder pedal assembly for play and travel freedom. 
__ -Lubricate hinges and torque tube bearings and check for safety. 
__ -Check rudder pedal return springs for attachment.

PAGE 3

CABLES AND PULLEYS-COCKPIT:

__ -Check all cables for broken strands.
__ -Remove butt fairings and check top deck aileron pulleys for wear and security. 
__ -Check aileron pulleys at both ends of panel.
__ -Remove floorboards, check and lubricate pulleys.

FLIGHT CONTROL OPERATION:

__ -Check aileron, rudder and elevator controls from cockpit for smooth operation. 
__ -Check wheel for neutral position with control surfaces streamlined.
__ -Lubricate control wheel shafts, universals, shaft bearing and sprocket chain.

__ -Lubricate control column bearings.

TRIM TAB CONTROLS:

__ -Check stabilizer trim control for smooth operation. 
__ -Check indicator against stabilizer for proper position.

FUEL SELECTOR VALVE:

__ -Check fuel valves for smooth operation.
__ -Check placard for "On" and "Off" positions. Check valve for leaks.

LANDING GEAR:

__ -Shock cord for broken strands and elongation.

__ -Lubricate landing gear tie-strut pivot.

AXLES AND WHEELS:

__ -Remove wheels, wash, check and lubricate bearings. 
__ -Check brake shoes for wear and drums for scoring.  Lubricate brake arm pivots.
__ -Install wheel and axle nut only tight enough to remove end play.

__ -Inspect tail wheel and lubricate swivel and wheel bearings.

TIRES AND FAIRING:

__ -Check tires for 20 lbs. of air pressure. Replace tires that have cord showing.

__ -Check gear fairings for security and chafing.

LANDING GEAR:

__ -Hoist aircraft (by engine mount at firewall) and check gear bushings, vee bushings

are replaceable if worn.
__ -Check for skin wrinkles indicative of inside damage.

WING FITTINGS:

__ -With wing root fairings removed, inspect wing fittings with a flashlight and

magnifying glass for minute cracks in the ears.
__ -Check bolts to he sure there are no threads in bearing and bolts are properly

safetied.

__ -Check wing fitting holes for elongation by pulling up and down on wing tips.

PAGE 4

LANDING GEAR FITTINGS:

__ -Remove both landing gear fairings and inspect all fittings with flashlight and

magnifying glass for signs of cracks or hole elongation.  Lubricate landing gear

pivots.

FUSELAGE STRUCTURE:

__ -Through inspection openings and through the baggage compartment cover, check

the condition of all tubing for rust, damage and protective coating.
__ -Check all wood stringers for damage and security.

DEBRIS ACCUMULATION:

__ -Check the bottom of the fuselage and fabric under floor boards for bolts, nuts and

other objects that might jam controls or pulleys.
__ -Check the rear of fuselage for open drain grommets.
__ -If considerable dirt or oil exists on the fuselage bottom use a non-caustic soap and

wash out the dirt to prevent fabric rot.

CONTROL CABLES AND PULLEYS:

__ -Check for broken control cable strands by sliding a cloth over the cable in vicinity of

fairleads.
__ -Check upper and lower elevator turnbuckles for safety and maximum of three

threads showing outside of barrel.
__ -Check stabilizer control for slippage. Increase tension by adjusting cable.

FAIRINGS:

__ -Check all fairings for cracks and missing screws.

WINGS AND AILERONS:

__ -Wing fabric: Check left and right wing fabric for holes, cracks or checks in the finish

and open drain grommets at each rib bay trailing edge. Fabric usually deteriorates

on the upper surface of the wing or along the trailing edge.
__ -Inspect drag wires for tension and wing ribs and compression members for damage.

STRUTS - LIFT:

__ -Check right and left wing strut fittings for elongation by having some one lift up and

down on the wing.
__ -Check bolts for fitting attachment to the spar.
__ -Check struts for dents or cracks, also sight down strut trailing edge to ascertain that

struts are straight.
__ -Check strut end forks and fork lock nut.

WING BOLTS:
__ -Check strut attachment bolts to be sure there are no threads in bearing, nuts are

not bottoming on unthreaded part of bolt and bolts are properly safetied.

PAGE 5

AILERONS:

__ -Check both ailerons for wrinkles which are possible signs of structural damage.

__ -Check each rib bay for an open drain grommet.
__ -Check condition of fabric and finish, refinishing any finish cracks, checks or

ringworm.

AILERON HINGES:

__ -Check aileron hinge legs for security at rear spar and false spar.
__ -Check hinge pins for wear and safety, lubricate. Worn or loose pins must be

replaced.

AILERON CONTROLS:

__ -Remove inspection covers and check the two cables in each wing for interference

and chafing.
__ -Check and lubricate aileron bellcrank bearings.

__ -Inspect and lubricate aileron push rod ends. 
__ -Check the four aileron horn bolts for wear, threads in bearing and safety.
__ -Check the six turnbuckles in the center top of fuselage for safety and not more than

three threads showing outside the barrel.
__ -To locate broken strands at fairleads or pulleys, slide a cloth over the cable. All

cables with broken strands are to be replaced.
__ -Check travel, 23 degrees up, 23 degrees down.

WING ROOT FAIRINGS:

__ -Check left and right wing root fairings for tension.
__ -Check all metal screws for security and the fairing for cracks.

STABILIZER: 

__ -Check stabilizer fabric condition and drain grommets for restrictions. 
__ -Lift up and down on the stabilizer checking for excessive play.

FIN:

__ -Inspect vertical fin for fabric condition and finish.

__ -Check for wrinkles, dents and signs of internal damage.

RUDDER:

__ -Inspect the fabric cover on the rudder for fabric and finish condition.
__ -Check bottom of rudder for an open drain grommet.
__ -Check rudder for alignment and possible internal damage usually indicated by a

wrinkle in the fabric.
__ -Inspect rudder hinge pins for wear and safety.  Lubricate.
__ -Check hinge bushings for play. These bushings are pressed in and should be

replaced when worn.

__ -Lubricate rudder control cable ends.
__ -Check rudder travel, 26 degrees left, 26 degrees right.

PAGE 6

ELEVATORS:

__ -Check fabric condition and finish on the elevators.
__ -Check for open drain grommets along the elevator trailing edge.
__ -Sight one elevator against the other for alignment.
__ -Inspect elevator hinge pins and bushings for wear and safety.  Lubricate.

__ -Check hinge bushings.  Replace any worn pins or bushings.

__ -Check elevator cable horns for safety, worn bolts and clearance in travel.

__ -Lubricate elevator control cable ends.

__ -Check elevator travel, 27 degrees up, 25 degrees down.

__ -Check and lubricate elevator trim tab hinges, linkage and trim tab oil hole.

EXTERNAL BRACING: 

__ -Check empennage rigging wires for corrosion and cracks or nicks that might result in

failure.
__ -Check fittings for alignment with the wire and check bolts for safety.
__ -Rigging wires should be taut with little hand deflection.
__ -Check each wire to be sure there are no loose fork lock nuts.

RUDDER AND ELEVATOR CONTROLS:

__ -Check rudder and elevator horns for worn bolts and safety with no threads in

bearing. --Check horns for alignment with the cable and freedom of travel.
__ -Check top and bottom cable turnbuckles for safety and a maximum of three threads

showing outside the barrel.
__ -Sight the cables through the fuselage for interference and chafing.

FAA REQUIREMENTS:

__ Check all Airworthiness Directives for applicability and compliance. 
__ Check for presence of Certificate of Airworthiness form.
__ Check for presence of Certificate of Registration form.
__ Check for Operation Limitations form.

__ Check for Weight and Balance form.
*Above items are required in cockpit when aircraft is currently licensed*

__________________

Current Airworthiness Directives:

__ 2008-09-18 – Wing Strut Attach Fittings – Service Bulletin 2007-02

__ 2008-04-09 – Left & Right Wing Front & Aft Lift Struts - Service Bulletin 2007-01

X  87-03-08 – Oil Pressure Gauge Hose

X  78-20-11 – Aileron Control Malfunction – Service Bulletin 78-002

X  51-09-03 – Fuel Shutoff Valve Clip – Service Bulletin Number 66

X  50-41-01 – Elevator Horn Bolt – Service Bulletin Number 65

__ 47-16-03 – Wing Strut Fittings

X  47-13-02 – Fuel Hose – Service Bulletin Number 60

Special Airworthiness Information Bulletin:

__ CE-11-05 – Main Landing Gear Tie Strut - Service Bulletin 78-001

PAGE 7