# Taylorcraft Foundation News Winter 2006 by Forrest Barber 12-19-06

Winter has not arrived in NE Ohio at this point. I have been flying the B Model on wheels, should have switched to pontoons!  Scott Taylor has elected to put a short mount   85 HP on his BC12D, so we will have good paper work on that project soon.

We seem to have a lot of new folks joining up and asking the usual initial questions and I try to steer them to the web site with its many answers. Today I hope to clear up a bit about the Airworthiness Directives as they apply to Taylorcraft:                UNDERLINED CAPS ARE MY COMMENTS!

47-13-02 TAYLORCRAFT:  Applies to Models BC-65, BCS-65, BC12-65, BCS12-65, BC12-D, BCS12-D, BCS12-D1 Aircraft. Inspection required each 25 hours of engine operation. This inspection applies only to fuel hose bearing white dash lines and having end fittings marked "CAA, SNA (date).  THESE HOSES HAVE NOT BEEN SEEN SINCE THE 1950’S.

47-16-03 TAYLORCRAFT: Applies to All Models BC, BF, and BL Series Aircraft. 
Compliance required immediately. Inspect wing strut attachment fittings on lower fuselage longerons for cracks or evidence of poor weld. If cracks or defects are found, the fitting should be replaced or reinforced. ERRORS MADE AT FACTORY AND FOUND ONLY ON FLOAT SHIPS, STILL GOOD IDEA TO INSPECT IF ABUSED.

50-41-01 TAYLORCRAFT: Applies to All Model B Series Aircraft, Serial Numbers 1001 and Up. Compliance required not later than November 15, 1950.

COMPLETE WORDING OMMITTED FOR SPACE, JUST DO IT!     INSPECT EACH YEAR TO SEE THAT THE BUSHING IS IN PLACE AND THERE IS NO INTERFERENCE

51-09-03 TAYLORCRAFT: Applies to All Series Aircraft Below Serial Number 13001 Incorporating a Pushpull Type Fuel Shutoff Control. Compliance required not later than June 1, 1951. To prevent accidental operation of the fuel shutoff valve during flight, a safety device must be installed on this control SEE THAT THE CLIP B12-947-3 OR EQIVALENT IS INSTALLED. NOTE; F-19 & UP HAD THIS INSTALLED AT THE FACTORY AT TIME OF MANUFACTURE

78-20-11 TAYLORCRAFT: Applies to Model BC-12D, Serial Nos. 6923, 8376, 8444, 8570, and 10538, and Model F-19, Serial Nos. F-080 through F-105, F- 107 through F-112 and F-115 through F-126 certificated in all categories. To prevent malfunction of the aileron controls, accomplish the following: WORDING OMMITTED FOR SPACE

THIS A.D. WAS CAUGHT IN THE VERY BEGINNING OF THE SALE BY FACTORY OF SPROCKETS WITH “SOFT” PINS.   CALL ME WITH QUESTIONS!!

87-03-08 TAYLORCRAFT: Amendment 39-5521. Applies to the following models and serial number airplanes certificated in any category:  WORDING OMMITTED.  THIS WAS PRECIPITATED BY THE LYCOMING ENGINE HIGHER OIL PRESSURE THAN THE CONTINENTAL BUT WE AT THE FACTORY SAW FIT TO CHECK ALL HOSES FOR COMPLIANCE.

Well that was quick and painless; I hope it adds a bit of understanding to these A.D. notes.   REMEMBER the Taylorcraft is a ship that has very few A.D.’s ; lets keep it that way.  Next time I will expand upon items to check that have failed from misuse of the aircraft.  These could lead to A.D.’s  bye Forrest A. Barber, President, Taylorcraft Foundation, TF#1, A&P, I.A.,   www.taylorcraft.org    www.barberaircraft.org